# Keystone RV Company	Page 1

2/14/2022

# Pre­Authorization

# 01:28:40 pm

Pre­Authorization numbe:r PA03044203 Reference no.: 66467	Status: Approved	Approved by: WillT	Approved date: 2/10/2022

Servicing	Name	Address	VIN	Retail sold on

Dealer

1194	Basden's American RV	Basden's American RV	4YDTSGL2XN3106679

600 E Baseline Rd Evansville, IN 47725

| Nu mbe r of |  | Root operation |  | Description |  | Sublet |  | Sub hours |  | Approved hours |  | Actual hours |  | Qty |  | Base hours |  | Complaint / Cause / Correction |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | 6103312A |  | Pump ­ Repair ­ E |  | 0.00 |  | 0.20 |  |  |  |  |  | 1 |  | 0.20 |  | Complaint:THE WATER PUMP BLOWS FUSES Cause:FOUND THAT YOU DIDNT WINTERIZE THE CAMPER HAD TO LET THE ICE MELT Correction:The water pump does not run. Replaced fuse and when pump is turned "ON", fuse blows. Removed water line on out side of water pump. Found "ICE" inside water pump. Used heat gun to melt ice in pump for 15 mins. KRV – The issue submitted is not considered a warranty defect in materials or workmanship. |
| 2 |  | 1210324B |  | Stabilizer Jack ­ H |  | 0.00 |  | 0.10 |  | 0.10 |  |  |  | 1 |  | 0.10 |  | Complaint:THE JACK HANDLE IS MISSING. THIS WAS NOTED ON DELIVERY. Cause:PART MISSING Correction:Replaced missing jack crank handle. |
| 3 |  | 6201224B |  | Rubber Hose ­ Re |  | 0.00 |  | 0.10 |  | 0.10 |  |  |  | 1 |  | 0.30 |  | Complaint:THE SPRAY PORT HOSE IS MISSING. Cause:PART MISSING Correction:Place new spray port hose in storage compartment. |
| 4 |  | 4131814B |  | Water Heater ­ Su |  | 0.00 |  | 0.80 |  | 0.20 |  |  |  | 1 |  | 0.20 |  | Complaint:THE WATER HEATER IS NOT WORKING ON LP Cause:IGNITER TOO CLOSE NOT LETTING IT FIRE Correction:Tried to light water heater on LP thru 5 cycles. Furnace next to runnig fine. Removed igniter and burner tube to check for obstructions. Burner tube ok. Igniter tips too close and not giving a good spark. Opened up gap on igniter. Remounted all parts. Checked operation again. Water heater ignited on first try. Burning OK. KRV – Labor time approved in accordance with Keystone RV’s flat rate manual. VCB: Vendor Charge Back item. (This is a Keystone RV internal notation only) |

Pre­Authorization numbe:r PA03044203	0.40

For paper claims only use one pre­authorization number per warranty claim for

Staple a copy of this pre­authorization to a warranty claim form. Fill out the header information and the parts section on the claim form. The labor section does not need to be filled out on the claim form. Fill in the amount of hours used for the repair ­ up to the amount authorized on the pre­authorization form and return toKeystone RV Company