# Keystone RV Company	Page 1

2/24/2022

# Pre­Authorization

# 01:27:00 pm

Pre­Authorization numbe:r PA03043702 Reference no.: 66470	Status: Approved	Approved by: holld	Approved date: 2/14/2022

Servicing	Name	Address	VIN	Retail sold on

Dealer

1194	Basden's American RV	Basden's American RV	4YDFHMU2XNA74107

600 E Baseline Rd Evansville, IN 47725

Nu mbe r of

Root operation

Description	Sublet Sub hours Approved hours

Actual hours

Qty		Base hours

Complaint / Cause / Correction

1 2105415B Slide Out ­ Box ­	0.00	18.00	12.00		1	0.50 Complaint:FOLLOWING ICE AND SNOW THERE WAS A

PUDDLE OF WATER NOTICED IN THE KITCHEN FLOOR

Cause:with the water in the location it was i checked the roof and found that it was leaking at the outer flange under the seam tape. the roof decking is lumped up from water damage and i found the outer flange had screws broken in it and the rubber roof does not even go under the flange	so if water gets past the seam tape it goes directly to the wood decking. with the leak in the slide out the water fell down the wall to the left of the fridge and damaged the wood on the inside and outside of the wall and possibly the framing depending on how long it has been leaking from the start	only leaks with the slide room out

Correction:replace the roof in the slide out, replace the decking in the slide out, replace the wall panel to the left of the fridge (outside and inside where the fridge is ) replace the fascia on the panels

KRV­ 10 hours for the slide roof and 2.0 hours for paneling that got damaged

Pre­Authorization numbe:r PA03043702	12.00

For paper claims only use one pre­authorization number per warranty claim for

Staple a copy of this pre­authorization to a warranty claim form. Fill out the header information and the parts section on the claim form. The labor section does not need to be filled out on the claim form. Fill in the amount of hours used for the repair ­ up to the amount authorized on the pre­authorization form and return toKeystone RV Company